- DMR Radio RT83 firmware Update

RT83-GPS.bin（GPS version ）

RT83-NOGPS（Non-GPS version）

- Firmware Solved the question

Solve the question about the key 0 doesn’t work when edit the messasge.键

- Software of Upgrade the firmware

Sigle Line Upgrade2_3.exe

- How to upgrade the firmware?

- Click to install the software of Upgrade the firmware “Sigle Line Upgrade2_3.exe”

- After, find and open the Application Program “UpgradeTools”, it will show the below:

- Connecting the radios to the computer via programming cable.

- Power off the radio, press the sidekey1 and sidekey2 at the same time, power on the radio( the indicator light will show red)

- In the firmware upgrade software, “Fimware file”choose the right version of firmware, Click “Search”and choose the right port in “COM Port”,"Baud rate" choose 57600

- Click“Write”to upgrade

- After successful, power off the radio, then power on again, the firmware is updated already.